Differential Sparse Regularization for Longitudinal Power Anomaly Localization

Scheme Assessment and Simulation Reproduction Report — from Theory to Simulation Results

August 10, 2026 · repo longrain153/PPE2 · branch claude/ppe-localization-accuracy-9tirgk

# 1. Executive Summary

This report assesses a "differential sparse PPE" scheme intended to improve fault-localization accuracy in fiber-longitudinal power profile estimation (PPE / Longitudinal Power Monitoring, LPM): build a low-noise reference profile from abundant pre-fault data, then, after the fault, solve only for the change relative to that reference under a step-sparsity prior (generalized Lasso / total-variation regularization). Theory and numerical simulation show the core idea is sound and is essentially the approach published by Fujitsu (OECC 2024) and NTT (ECOC 2025). Under 64 GBd, 3 × 50 km, a 1-dB lumped loss, and a single post-fault capture, localization error improves from tens of kilometres (naive profile subtraction) to about 1 km — one to two orders of magnitude. However, two formulation details in the original proposal require correction, and the claimed "50–200 m, purely noise-limited" accuracy is over-optimistic: the spatial-resolution physics set by chromatic dispersion × bandwidth² still applies, and kilometre-scale accuracy is the realistic expectation in this regime, consistent with experimental literature.

# 2. Background and Problem

PPE/LPM inverts the Kerr nonlinearity imprinted on the received signal to reconstruct the power distribution along a link using receiver-side DSP only — no OTDR or dedicated hardware. It can monitor amplifier gains, span losses, and anomalous loss events (degraded splices, cable stress). The core challenge is that under practical low-power, limited-data conditions the estimate is noisy and spatially resolution-limited, making small lumped losses hard to pinpoint.

The operational scenario considered here: before the fault there is ample time to collect data and form a reference; after the fault, data are limited, and a single dominant loss event must be localized as accurately as possible.

# 3. Theory

## 3.1 The LS-PPE linear model (RP1)

Under the first-order regular perturbation (RP1) approximation, the nonlinear perturbation of the received signal is linear in the "generalized nonlinear coefficient" γ′(z) = γ·P(z). Discretizing the link into M spatial bins gives the matrix form:

A₁ ≈ G·γ′,  gₖ = D(−zₖ)[ j·|uₖ|²uₖ ],  uₖ = D(zₖ)·u₀

where D(z) is the dispersion operator and u₀ the known transmitted waveform. Each column gₖ is the receiver-side "fingerprint" of nonlinearity generated at position zₖ. The least-squares solution (LS-PPE, Sasai et al., JLT 2023) is:

γ̂′ = ( Re[G†G] )⁻¹ Re[G†A₁]

## 3.2 Ill-posedness and the minimum step size

Adjacent columns gₖ, gₖ₊₁ are distinguishable only through the waveform change that chromatic dispersion produces between the two positions. When the step Δz is too small, adjacent columns become highly correlated, Re[G†G] approaches singularity, and noise is severely amplified. The smallest resolvable step is set jointly by dispersion and signal bandwidth:

Δz_min ∝ 1 / ( |β₂|·BW² )

This is a physical limit: regularization does not change the physics behind the column correlation; it stabilizes the inversion mathematically by trading bias for variance through prior knowledge.

## 3.3 The differential sparse scheme and two required corrections

The scheme under assessment: with reference profile x_ref (averaged from abundant pre-fault data), exploit model linearity to solve for the change Δx after the fault, with an L₁ sparsity constraint on its spatial derivative so the entire power drop collapses onto a single change point. Two details had to be corrected:

- Correction 1: Δy = y_ref − y_fault is not directly computable. Different captures carry different transmitted symbols, so perturbation vectors from different captures cannot be subtracted sample-by-sample. The correct differential residual projects the reference profile through the current capture's own operator: r = y_fault − G_fault·x_ref.

- Correction 2: a plain first-difference penalty ‖DΔx‖₁ is the wrong sparsity basis. Δx itself decays with fiber loss and jumps at EDFAs, so its plain first difference is not sparse. The total-variation constraint must be applied in the relative-change domain u = Δx / x_ref, where a lumped loss makes u an exact step function (equivalent to Fujitsu's attenuation-weighted J-matrix).

The corrected optimization problem is:

min_u ½‖ G·diag(x_ref)·u − r ‖₂² + λ‖ D u ‖₁

## 3.4 Solver and refinement

- ADMM + λ path: The generalized Lasso is solved with ADMM; λ is swept along a descending, warm-started path, keeping the solution with the sparsest non-empty jump support — no manual tuning. Support detection uses ADMM's sparse copy variable, which is exactly sparse.

- Matched-filter refinement: Centred on the sparse-detected bin, candidate fault positions are scanned on a 100-m grid: the RP1 step signature of a unit-depth loss at z_c, s(z_c) = −G·[x_ref·step(z>z_c)], is correlated with the residual r; the correlation peak gives the position, and a least-squares amplitude refit removes the Lasso shrinkage bias to estimate the loss magnitude.

# 4. Simulation Setup

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
| Signal | 64 GBd 16QAM, RRC 0.1 | Link | 3 × 50 km SSMF |
| Loss / dispersion | 0.2 dB/km, β₂ = −21.4 ps²/km | Nonlinear coeff. | γ = 1.3 W⁻¹km⁻¹ |
| Launch power | 6 dBm | Noise | EDFA ASE (NF 5 dB) + 18-dB Rx SNR |
| Per capture | 8192 symbols | Propagation | Split-step Fourier, 100-m steps |
| PPE grid | Δz = 1 km (150 bins) | Refinement grid | 100 m |
| Fault | 1.0-dB lumped loss @ 72.35 km (off-grid) | Data | 12-capture reference; 1 post-fault capture |

Three localization methods are compared: (a) naive baseline — subtract the single-capture post-fault LS profile from the reference and take the largest drop of the relative-change difference; (b) differential sparse (corrected generalized Lasso, 1-km grid); (c) matched-filter refinement of (b) on a 100-m grid. Four independent fault trials (different data and noise realizations).

# 5. Simulation Results

## 5.1 Reference profile (healthy link)

Fig. 1  Healthy-link power profile: single-capture LS-PPE (blue) is visibly noisy; the 12-capture average (red) serves as the reference, with the three-span sawtooth structure clearly resolved.

## 5.2 Differential sparse localization

Fig. 2  1-dB lumped loss, single post-fault capture. Top: naive subtraction (blue) is buried in noise, while the differential sparse solution (red) is a clean step at the right position (amplitude shrunk by the Lasso; refit in Sec. 5.3). Bottom (twin axes — the two indicators differ by ~3 orders of magnitude): the naive jump indicator (blue, left axis, ±25) fluctuates along the whole link; the sparse jump indicator (red stems, right axis, ±0.06) is nonzero in only 2 of 149 bins, both clustered at the fault, and its deepest jump gives the detected position 73.5 km (true: 72.35 km).

## 5.3 Matched-filter refinement and quantitative results

Fig. 3  Matched-filter correlation on the 100-m grid (trial 1): the correlation surface is nearly flat over several km — adjacent candidate signatures decorrelate only through chromatic dispersion, so the km-scale ambiguity is the bandwidth/dispersion physics limit reasserting itself.

| Localization method | Mean abs. error | Max abs. error | Notes |
| --- | --- | --- | --- |
| Naive profile subtraction (LS) | ≈ 40 km | 75 km | Noise-dominated, unusable |
| Differential sparse (1-km grid) | 0.9 km | 1.15 km | Jump support concentrated at the fault |
| Matched-filter refinement (100-m grid) | 1.5 km | 4.2 km | Amplitude refit 0.85–0.95 dB (true 1.0 dB) |

In all four trials the differential sparse method places its jump support at the fault bin (errors 0.15–1.15 km, near the 1-km grid resolution). The matched filter refines to 150–350 m in some trials but drifts up to 4 km in others — the finer grid does not deliver reliable sub-km accuracy, confirming the flat correlation surface of Fig. 3. The loss magnitude is accurately recovered by the refit (0.85–0.95 dB).

# 6. Verdict on the Original Claims

| Claim | Verdict | Basis |
| --- | --- | --- |
| Δz_min is set by dispersion and bandwidth² | Correct | Consistent with Sasai's ill-posedness analysis; also seen in the flat correlation surface |
| Differential + sparsity prior greatly improves localization | Correct | Tens of km → ≈ 1 km, one to two orders of magnitude |
| Pre-existing loss events cancel in the differential; only a small shadowing term remains | Correct | Follows directly from model linearity |
| Subtract measurements directly: Δy = y_ref − y_fault | Needs correction | Different captures carry different symbols; use r = y_fault − G·x_ref |
| Plain first difference of Δx as the sparsity basis | Needs correction | Apply TV in the relative domain u = Δx/x_ref (attenuation-weighted) |
| 50–200 m accuracy, purely noise-limited | Over-optimistic | The physics limit remains; this simulation and the literature (NTT +3 km, Fujitsu <4 km, CICT 1 km) are all km-scale |

Overall conclusion: differential sparse PPE is a correct and worthwhile engineering approach. Its gain comes from degrading the full-profile reconstruction problem into a single-parameter (step-position) detection problem, thereby exploiting the information of every bin downstream of the fault. Its accuracy ceiling, however, is still bounded by the dispersion × bandwidth² resolution physics: at 64–100 GBd, roughly 1 km — not hundreds of metres — is the realistic expectation.

# 7. Reproduction Code and References

Code lives in repo longrain153/PPE2 (branch claude/ppe-localization-accuracy-9tirgk): ppe/link_sim.py (split-step link simulator), ppe/ppe_core.py (G matrix, LS-PPE, ADMM generalized Lasso, matched filter), run_experiment.py (end-to-end experiment, ~2.5 min).

- T. Sasai et al., “Linear Least Squares Estimation of Fiber-Longitudinal Optical Power Profile,” J. Lightwave Technol., vol. 42, no. 6, 2023.

- R. Shinzaki et al. (Fujitsu), “Sparse Modeling Analysis for Efficient Localization of Power Anomalies in Transmission Links,” OECC 2024.

- H. Ishihara et al. (NTT), “Robust Fibre Longitudinal Power Monitoring with Few Measurements using Two-stage Sparse Regularization,” ECOC 2025, W.01.06.4.

- Y. Zuo et al. (CICT), “High-fidelity fiber longitudinal power monitoring via non-uniform sparse regularization,” Opt. Express, vol. 34, no. 10, 2026.